ST1100 ABS Sensor replacement.

This is fairly easy to do but some care is required to prevent damage to the new sensor.

- Remove the faulty sensor and cut off the connector along with 20cm of cable.

- Solder or connect the old plug onto the new sensor wire, matching the colours and put on a small piece of the supplied heat shrink sleeving to cover the joint.

- Install the new sensor into your mounting bracket using the small washers supplied as spacers between the new sensor and the mounting. This should gap the sensor almost in the correct position.

- Route the cable as required.

- The sensor will work over quite a large range but you should check the gap distance and that the sensor isn’t touching the toothed wheel.

- The sensor can be checked for correct operation by measuring the signal between the green and blue wires. The voltage should vary between about .5V and 4V when the wheel is turned (and the ignition is on).

If you have any issue with the sensor or require help

please contact us on 0796-345-9885